Preparing the BPQ configuration file for the Kantronics Data Engine

In order to make a G8BPQ EPROM for your Data Engine, you must first

edit the file BPQCFG.TXT using a word processor. Be sure to be in the

text (non-document or ASCII) mode of the word processor. Many of the

items listed in the BPQCFG.TXT file are not used in the Data Engine

version, but must be present in the file in order to properly compile.

There are three basic steps involved in making your EPROM. First,

you must edit the BPQCFG.TXT file. Secondly, you run the program

BPQCFG.EXE, which compiles the TXT file into a binary image, using the

parameters you have specified. This produces a file named BPQCFG.BIN.

Third you run the program KMERGE.COM, which will combine the basic BPQ

code with your specific configuration file, resulting in a file called

KANT.PRM which can now be burned into a 27C512 EPROM for use in the

Data Engine. It may be placed in either EPROM socket (U-7 or U-8).

If you place the BPQ EPROM in U-8, you must leave the original

Kantronics EPROM in U-7. This allows you to select the Kantronics

end-user firmware by turning the power on with the AUX switch OUT,

or selecting the BPQ EPROM by depressing the AUX switch before

applying power to the Data Engine. Once you have the Data Engine

power up with the BPQ code, the AUX switch selects the serial port

configuration.

This document describes the entries of the BPQCFG.TXT file, with

guidelines for editing it to match your configuration needs. Within

the BPQCFG.TXT file, any line beginning with a semi-colon (;) is a

comment line, and is ignored by the compile program.

HOSTINTERRUPT	This sets the software interrupt used in PC based

versions of the BPQ code - it is not used in the Data Engine, but

must be present. Leave it set to 127.

EMS  This determines whether or not the BPQ code will use Expanded

memory - since the Data Engine does not have expanded memory, this

should be set to 0.

DESQVIEW  This parameter is used with the PC version to indicate that

the system is running DesqView. For the Data Engine, this parameter is

ignored, but should be set to 0.

NODECALL  This sets the callsign used for the node. Enter your

callsign (with SSID if desired). Users may connect to the NODECALL to

access the node.

NODEALIAS This sets an alias, or secondary name, for the node. Users

may also connect to the node using the alias. In addition, the user

may connect to any SSID of the NODEALIAS.

BBSCALL   Since the Data Engine does not have a BBS directly

associated with it, this must be set the same as the NODECALL

(including SSID).

BBSALIAS  Again, since there is no BBS, set this the same as the

NODEALIAS.

IDMSG     This section contains the text that will be transmitted as

the ID packet from the node. If the first line of the text is as shown

in the default (Network Node and alias in parentheses) then the KA-

NODES in your area will also list your node in their nodes heard list.

This may be up to 512 characters long and you must end it with the

three *** on the next line.

UNPROTO   This sets the destination callsign for un-connected packets

transmitted from the node. (Typically this may be set to CQ, or your

name).

INFOMSG   This section is the text transmitted to a user who has

connected to the node and issues the INFO command. It can be a

description of your node configuration, equipment used, etc. Maximum

length of the INFOMSG is 512 characters, and you must end it with ***

on the next line.

CTEXT     This is the connect text, sent to a user who connects to the

NODEALIAS. This is not sent when a user connects to the NODECALL, or

if the connect is from another NODE.

OBSINIT   This is the initial value of the obsolescence counter. The

initial value is assigned to a neighboring node whenever your node

copies the other node's NODES broadcast. Each time your node does it's

own NODES broadcast, the obsolescence count is decremented. When the

obsolescence count falls below the OBSMIN value for a given

neighboring node, that node will no longer be included in your NODES

broadcast.

OBSMIN    This sets the minimum value of the obsolescence counter

allowed for a node to be included in the NODES broadcast.

NODESINTERVAL  This value determines how often your node will

broadcast its NODES table (in one minute increments). This should be

standardized for all nodes in your area.

IDINTERVAL     This sets the time between ID broadcasts from your

node. FCC regulations require an ID every 10 minutes.

BTINTERVAL     This sets the time interval between BEACONS from your

node. This should be set to 0 in the Data Engine code, as it is

intended for the PC based version to allow W0RLI BBS systems to send a

mail beacon periodically.

L3TIMETOLIVE   This controls the maximum number of hops that a single

packet may travel in the network. If the packet has not reached its

destination within that number of hops, it is probably in a circular

path, and will then be discarded by the node which decrements its TTL

counter to 0 (Time-to-live).

L4RETRIES      This sets the number of retries on a Level 4 packet

(transport layer - used between nodes).

L4TIMEOUT      This value sets the timeout (in seconds) for level 4

packets.

L4DELAY        This value controls the time interval between receiving

a level 4 packet, and sending the ACK for that packet (in 1 second

intervals). The delay may allow the ACK to be piggy-backed with

information headed in the opposite direction.

L4WINDOW       This value sets the "maxframe" for level 4 information

between nodes. It is negotiated between your node and the node you

connect to at connect time. (Smallest value wins).

MAXLINKS       This sets the maximum number of level 2 (standard AX.25

end user) links that your node will allow at one time.

MAXNODES       The maximum number of nodes that will be listed in your

NODES table. This is the list sent to a user who connects to your node

and issues the NODES command.

MAXROUTES      This sets the maximum number of nodes you may list as

adjacent nodes - that is those you can hear directly.

MAXCIRCUITS    The maximum number of level 4 circuits that may exist

through your node at any given time.

MINQUAL        The minimum calculated path quality necessary for a

node to be listed in your NODES table.

BBSQUAL        Since there is no BBS, this setting doesn't make any

difference.

BUFFERS        This sets the number of packet buffers available to the

node. In the Data Engine, you should leave this set to 255 (allocate

as many buffers as possible).

PACLEN         This sets the maximum data length of a packet sent from

your node to another node. In order to avoid having to fragment

packets and then re-form them, this value should be 236 or less.

TRANSDELAY     This is equivalent to the TNC parameter PACTIME. It

sets the delay (in 1 second increments) before the node will send data

after the last characters have been received from the serial port.

T3        This timer is equivalent to the TNC CHECK parameter. If no

data is passed through the link for T3 time, then a poll will be

performed to insure the link is still valid.

IDLETIME  This parameter set the interval (in seconds) for a shutdown

of the existing link. If no data is received for a period of IDLETIME,

then the node will assume that the link is no longer in use, and will

disconnect the link.

BBS       This parameter selects whether or not to include BBS

support. In the Data Engine, it should be set to 0.

NODE      This parameter selects whether or not to allow the node to

operate as a full packet switch. You will want this selected in the

Data Engine to allow it to automatically connect to other nodes as

part of the network.

HIDENODES When set to 1, any node whose NODEALIAS begins with a # will

not be displayed to a user who issues a simple NODES command. In order

to see the "hidden" nodes, the user must enter the NODES * command. If

set to 0, all nodes will be displayed with the NODES command.

ENABLE_LINKED When set to Y, enables a *** LINKED command to be

used by any user. If set to A, allows BPQ applications to use

the linked commands, and if set to N disables the *** LINKED

command completely. For the Data Engine, this should be either Y

or N.

Next in the file is the TNC port definitions. This configures the

serial port of the Data Engine to appear as a "standard" TNC2 when the

BPQ code is running and the AUX switch of the Data Engine is in the

OUT position. This allows you to connect to the node with a dumb

terminal or standard communications program on your computer. Since

there is only one serial port on the Data Engine, you must configure

only one TNCPORT. You should configure it exactly as follows:

TNCPORT

COM=1

APPLMASK=2

ENDPORT

The COM=1 statement tells the code how to talk to the serial port. The

APPL=2 statement sets up the serial port so that a user who connects

to the node can then connect to your terminal by issuing the CONNECT

command without any parameters. (Simply types C or CONNECT).

The next section of the Configuration file identifies the AX.25 ports

(actual radio connections). Each port begins with the word PORT at the

beginning of the line and ends with the word ENDPORT at the beginning

of a line. The fields in the port definition and their settings are

discussed below.

ID   This is the text sent to the user for each radio port defined.

This is sent when a user connects to the node and issues a PORTS

command.

TYPE This tells the BPQ code what type of hardware connection is

provided to the radio (for the Data Engine it must be DE56). For those

ports which are connected through the serial port of the Data Engine to

another TNC, you would set the TYPE=ASYNC.

PROTOCOL  This field specifies the type of interface between the BPQ

code and the radio. HDLC specifies the radio ports of the Data Engine.

If set to KISS, this would indicate that the connected radio is using

a TNC in KISS mode to talk to the BPQ system. This may be used to

allow the serial port of the Data Engine to communicate with another

TNC to support additional radios.

KISSOPTIONS  This parameter tells the BPQ code which type of KISS

interface to use for a TNC connected to the serial port. (You must have

PROTOCOL=KISS to specify this parameter). If set to CHECKSUM, each packet

sent from the Data Engine to the TNC will use a checksum method to prevent

errors. If set to POLLED, the other TNCs connected to the Data Engine will

not send any data to the Data Engine until they are polled. If set to

ACKMODE, the attached TNCs will send an acknowledgement back to the Data

Engine when the data has actually been transmitted. (Note that these modes

require SPECIAL KISS Eproms. Kantronics has the special KISS eproms

available as part this distribution for the KAM, KPC-4, KPC-2, KPC-2400

and KPC-1).

IOADDR    This specifies the address used to talk to the radio. In the

case of the Data Engine RADIO ports, the proper value is 0H. When

setting up a PORT to operate through the serial port of the Data

Engine to another TNC, the IOADDR should be 0E0H.

INTLEVEL  This specifies the INTERRUPT used for the I/O channel. On

the Data Engine, the proper INTLEVEL for the RADIO ports is 3, and the

serial port uses INTLEVEL 1.

SPEED     This should be set to indicate the speed of the RADIO port

(1200 baud for instance). Four modems are currently available inside

the Data Engine - 1200 baud (DE1200), 2400 bps (DE2400), 9600 baud

DE19K2/9K6) and 19200 baud (DE19K2/9K6). It is possible, however to

use other modems (HAPN, PSK, etc), so match the proper baud rate for

your modem. For the serial port match the SPEED to the serial port

baud rate of the other TNC. If using the Kantronics DE2400 or

DE19K2/9K6 modem, or any other modem that supplies all the clock

signals, set SPEED=0.

CHANNEL   This parameter allows the code to access more than one radio

port at the same IOADDR and INTLEVEL. For the Data Engine, Radio port

1 is CHANNEL A, Radio port 2 is CHANNEL B. Likewise, for a KPC-4 in

KISS mode (as an external TNC) Port 1 is CHANNEL A and Port 2 is

CHANNEL B.

INTERLOCK  This parameter allows you to prevent multiple radio ports from

keying their radios simultaneously. All ports with the same INTERLOCK

level are only permitted to transmit one at a time. For instance if Ports

1 and 3 have INTERLOCK=1, and Ports 2 and 4 have INTERLOCK=2, then Ports 1

and 3 will not transmit simultaneously, but 1 and 2 could transmit at the

same time.

QUALITY   This determines the path quality assigned to any other NODE

whose NODES broadcast is heard directly on this port. Values may be 1-

255, and should be coordinated with other nodes in your area.

MAXFRAME  This sets the maximum number of frames sent before waiting

for an ACK from the receiving end.

TXDELAY   Typical TXDelay setting as in your TNC (1 millisecond

increments)

SLOTTIME  Same as in TNC firmware, in 1 millisecond increments.

PERSIST   Same as in TNC firmware - persistence value.

FULLDUP   Not yet implemented - leave set to 0.

SOFTDCD   Not yet implemented - leave set to 0.

FRACK     Timer used when waiting for ack from other end. Failure to

receive ack within this time results in retry. (1 millisecond

increments.

RESPTIME  Length of time to wait after receiving packet before sending

the ack. If you connect the serial port of the Data Engine to another

TNC (in KISS mode) you should set the RESPTIME large enough to allow a

second packet to be received before sending the ACK. The default value

of 2000 (2 seconds) is sufficient.

RETRIES   Number of retries permitted on this port.

PACLEN    Maximum size of the data portion of a packet on this port.

CWID      Some countries require all stations to ID periodically in

CW. This parameter allows for such requirements. If you do not need

this, simply comment the line (put a ; at the beginning of the line.

DIGIFLAG  Setting the DIGIFLAG to 0 will not allow any digipeating

through your node. If set to 1, digipeating is permitted, and if set

to 255, digipeating is allowed only for UI frames (not connected).

DIGIPORT  It is possible to allow "cross-port" digipeating. Setting

this parameter to 0 causes digipeating to be on the same port it was

received. If set to a non-0 value (N), the packets received on this

port will be digipeated on port N.

USERS     This sets the maximum number of level 2 connects (end users)

allowed on this port at the same time. If set to 0, the number of

users is unlimited.

UNPROTO   Set the default UNPROTO path for this port. If digipeaters

are required, do not use the word via, simply specify your

DESTINATION, a space, and then DIGIPEATERS separated by spaces.

VALIDCALLS     This may be used to restrict any port to only those

users whose calls are specified in the VALIDCALLS. (All SSID's are

allowed if the call is present.) The total length of the validcalls

can be 255 characters, and it may be entered on multiple lines.

ROUTES    This section allows you to specify the routes (neighboring

nodes) that you wish to "lock" into the node. Enter the neighboring

node's CALL, QUALITY, and PORT NUMBER.

APPLICATIONS   This field is not used currently in the Data Engine

version, but must be present for the BPQCFG.EXE compiler to operate

properly.

After you have completed editing the file, you are ready to run the

BPQCFG.EXE program, thus creating your BPQCFG.BIN file. After you run

this program, next run the KMERGE program, which provides a file named

KANT.PRM on your disk. Now burn this image into a 27C512 Eprom and it

is ready for use in either EPROM socket on the Data Engine with front

panel selection of either the BPQ code or the standard Kantronics code.

Good luck.