SCALC

A powerful microwave S-parameter analysis and optimization program

that doesn't cost thousands of dollars.

CONTENTS

INTRODUCTION

1.  GETTING STARTED

1.1  Installation

1.2  Keyboard input conventions

2.  USING THIS MANUAL

PART A - NETWORK  ANALYSIS

3.  A TOUR OF SCALC

4.  MAIN MENU FUNCTIONS

5.  SIMPLE PASSIVE NETWORKS

5.1  How SCALC works

5.2  Simple R/C/L elements

5.3  Cascading elements

5.4  Combined R/C/L elements

6.  ADVANCED PASSIVE NETWORKS

6.1  Repeating networks using EQU and IMA

6.2  Transmission lines

6.3  Non-ladder networks using SER and PAR

7.  S-PARAMETER (2-PORT) FILES

8.  OTHER TYPES OF DATAFILES

9.  CODEWORDS CLASSIFIED

9.1  Introduction

9.2 - 9.5    See printed manual

(use on-line Help in unregistered version)

10.  NETWORK EDITOR

10.1  General description

10.2  Network Editor commands

10.3  Network checking

11.  DATAFILE EDITOR

11.1  One-port or two-port data

11.2  Two-port data format

11.3  One-port data formats

11.4  Graphical review

11.5  Saving data files to disk

12.  ANALYSIS - SETTING FREQUENCY RANGE

13.  RESPONSE MENU

13.1  Table of reflection S-parameters

13.2  Table of transmission S-parameters

13.3  Table of input/output impedances

13.4  Table of S-parameters in dB

13.5  Save S-parameters to data file

13.6  Table of input stability circles

13.7  Table of output stability circles

13.8  Smith chart of S12 and S22

13.9  Graph of reflection S-parameters

13.10  Graphs of transmission S-parameters (S21 and S12)

13.11  Main Menu shortcuts

PART B - OPTIMIZATION    in file SCALC-B.DOC

14.  ABOUT OPTIMIZATION

14.1  Introduction

14.2  Optimization overview

14.3  Steps in optimization

15.  OBJECTIVES EDITOR

15.1  Overview

15.2  A simple example

15.3  Syntax for objectives

15.4  Choice of frequencies

15.5  Leaving the Objectives Editor

15.6  Another example

16.  A SIMPLE OPTIMIZATION

17.  SENSITIVITY ANALYSIS

18.  OPTIMIZATION PROCEDURES

19.  FINDING THE OPTIMUM COMPROMISE

19.1  Two kinds of problem

19.2  Guiding the optimization

19.3  Say `No' to optimizer dependency

19.4  Good candidates for optimization

20.  OPTIMIZATION OF AN AMPLIFIER

20.1  Prototype design

20.2  Analysis

20.3  Stability circles

20.4  What to optimize?

20.5  Optimization results

CONCLUSION

SCALC ERROR CODES

INTRODUCTION

SCALC is a powerful microwave S-parameter analysis and optimization program

that doesn't cost thousands of dollars.

SCALC was developed by Roger Blackwell G4PMK and Ian White G3SEK, from the

Public Domain program ALMOND by Chris Trask, whose contribution is gratefully

acknowledged. We have improved on the original by rewriting and optimizing the

code, adding a civilized user interface and Help information, and producing

this new and complete manual.

SCALC is Shareware. There are plenty of further features that we'd like to add,

but if you want SCALC to continue to improve, please register to show your

support for future developments.

To register, please see the file REGISTER.DOC. You will receive:

* A registered copy of SCALC, and updates on new developments

* The complete printed user manual, with illustrated worked examples

* Lots of extra S-parameter files for useful microwave devices

* Interconversion utilities for PUFF and (P)SPICE 2-port device files.

Please note that the registration fee is only for the improvements that we

have made beyond Chris Trask's original PD program. If you don't register,

you really ought to be using ALMOND - and not SCALC with its improved features.

*******************************************************************************

Legalities

This software and manual are sold "AS IS" and without warranties, whether

express or implied. Because of the various hardware and software environments

into which this software may be put, no warranties of merchantability, fitness

for a particular purpose or functionality are offered. Good practice dictates

that any program be thoroughly tested with non-critical data before reliance is

placed on it. The user must assume the entire risk of using this software. The

liability of the seller will be limited exclusively to product replacement in

the case of faulty media, or to refund of the purchase price. This contract

shall be governed by English law.

The program SCALC and associated programs are copyright 1990-1995,

Roger Blackwell and Ian White. No claim is made on the portions of the

program that were originated as ALMOND by Chris Trask.

Portions of the executable program are copyright 1990, Microsoft Corporation.

This manual is copyright 1995, IFW Technical Services.

*******************************************************************************

1.    GETTING STARTED

1.1   Installation

Simply unzip the archive SCALC.ZIP and copy all the SCALC files to the disk

directory of your choice.

All files must be together in the same directory, but SCALC provides you with

sorted listings when needed.

1.2   Keyboard input conventions

Press, Type or Enter?

These three words are used consistently throughout this manual.

Press x    -Press the x key and release it before keyboard auto-repeat begins.

Type xyz   -Press x (as above), press y and then press z.

DON'T press the Enter key.

Enter xyz  -Press x, press y, press z and then press the Enter (or Return) key.

Input Prompts

Throughout SCALC, an input that does not require the Enter key ends with '>',

e.g.   Press any key >

An input that must be terminated by the Enter key ends with ':',

e.g.        Filename :

Default Keypress Options

Many messages in SCALC require a single keypress for 'Yes' or 'No', or similar

options. The capital letter shows the default - you can press ALMOST any key to

get this.

[Y/n] > Yes is the default. To choose No you must press either N or n;

any other keypress will be interpreted as Yes.

[y/N] > No is the default. To choose Yes you must press either Y or y;

any other keypress will be interpreted as No.

TIP

To accept a series of default options, just keep pressing the Enter key or the

spacebar.

2.    USING THIS MANUAL

The rest of this manual is divided into two major parts.

Part A - NETWORK ANALYSIS - comprises Sections 3-13. Section 3 gives you a brief

tour of SCALC's analysis functions. Sections 4-13 show you how to build up

networks for analysis by SCALC, starting with simple networks and progressing

until you can use all of SCALC's extensive range of network codes.

Part B - OPTIMIZATION - comprises Sections 15-20. It explains how to use

SCALC's powerful optimization facilities to develop networks which meet your

specified performance parameters.

On disk, Part B is the separate file SCALC-B.DOC

Character Set in Disk Manuals

SCALC uses the 'IBM PC' extended ASCII on-screen character set for its displays,

including the � and � signs, and also some line-drawing characters. To include

sample SCALC screens in the disk version of the manuals, we have used the same

character set. This also allows us to draw example circuits using real lines and

boxes rather than 'ASCII art'.

When viewing and printing the disk manuals SCALC-A.DOC and SCALC-B.DOC, make

sure your software and printer are configured to use the 'IBM PC' or 'PC-8'

character set.

PART A

NETWORK  ANALYSIS

Sections 3 - 13

3.    A TOUR OF SCALC

To run the program, change to the directory where all the SCALC files are kept,

and enter SCALC   The first menu you see is the SCALC Main Menu (Figure 1).

�����������������������������������������������������������������������������Ŀ

E - Edit network         F - Frequency range        A - Analyse network

O - Optimization         Q - Quit to DOS            I - Info on program

�������������������������������������������������������������������������������

Figure 1:  SCALC Main Menu

Press E to enter the Network Editor (Figure 2). Note the yellow Network Editor

display at the top right corner of the screen, which always shows whereabouts in

SCALC you are.

�����������������������������������������������������������������������������Ŀ

Line number        1                                           Network Editor

Element type

Element number

Element No.   #1       #2       #3    | Element No.   #1       #2       #3

I - Insert line   P - Print network  C - Clear network          _ - Line up

M - Modify line   S - Save network   E - Datafile Editor        _ - Line down

D - Delete line   L - Load network   F - Network file catalog   Esc - Done

Press F1 for help on element types

�������������������������������������������������������������������������������

Figure 2:  Network Editor

For this demonstration you are going to load an existing network and analyse it.

Optimization comes later, after you have learned how to write your own SCALC

network files.

Press L to load a network file, and enter LPF against the filename prompt. This

loads and displays the network file called LPF.NET. All of SCALC's network files

have the suffix .NET

CAUTION File suffixes are automatically assumed by SCALC, according to context.

NEVER enter any file suffix yourself.

The .NET file consists of a series of lines, each describing a circuit element

and its parameters. LPF.NET describes a simple 3-pole lowpass filter designed

for a cutoff frequency of 1.0GHz. Figure 3 shows the circuit and how each

component relates to a circuit element in the listing. The last two lines do not

relate to components as such: element 6 (CAX 1 5) tells SCALC to analyse the

whole circuit from element 1 to element 5 inclusive, and finally END 6 says that

element 6 was the last one in the circuit.

2 �������Ŀ         4 �������Ŀ

�����������Ĵ  10nH �����������Ĵ  10nH �������������

�     ���������     �     ���������     �

�����Ŀ             �����Ŀ             �����Ŀ

�     �             �     �             �     �

input  � 2p2 � 1           � 4p7 � 3         5 � 2p2 �    output

�     �             �     �             �     �

�������             �������             �������

�                   �                   �

�����������������������������������������������������

Element No.   #1       #2       #3

CPP   1    2.2pF

INS   2    10nH

CPP   3    4.7pF

INS   4    10nH

CPP   5    2.2pF

CAX   6    1el#     5el#

END   6

Figure 3:  Circuit and listing for LPF.NET

The Network Editor will let you scan through the lines using the down and up

arrow keys. It also lets you insert, change and delete lines as you wish - but

don't try to do that now. Section 10 gives further details about the Network

Editor.

For now, press Esc to leave this screen. SCALC checks the data for consistency

and won't let you leave the Network Editor until the file is correct. (If you

get into trouble because you couldn't resist editing the file, press L again and

reload the original file.) When the network has been checked successfully you

are returned to the SCALC Main Menu.

Press F to set up a frequency range for the analysis; the display at the top

right changes to Frequency Range. There are two ways to do this - manual entry

of up to 20 frequencies, or a sweep option. We will use the sweep option, by

entering a start frequency and a frequency interval.

Since the lowpass filter has a cutoff of 1.0GHz, you could sensibly start at

0.2GHz with a frequency interval of 0.2GHz. Enter these two values in response

to the prompts and SCALC will automatically allocate 20 analysis frequencies

ending at 4.0GHz. After a brief display of the 'stop' frequency SCALC returns to

the Main Menu.

Now press A to analyse the circuit at the chosen frequencies. The top-right

display changes to Response Menu. The Analysis in progress message continues to

flash until it changes to Analysis complete and the Response menu appears

(Figure 4).

�����������������������������������������������������������������������������Ŀ

1 - Table of reflection S-parameters     6 - Table of input stability circles

2 - Table of transmission S-parameters   7 - Table of output stability circles

3 - Table of input/output impedances     8 - Smith chart of S11 and S22

4 - Table of S-parameters in dB          9 - Graph of reflection S-parameters

5 - Save S-parameters to data file       0 - Graph of transmission S-parameters

F - Frequency range    O - Optimization     E - Edit network    Esc - Main Menu

�������������������������������������������������������������������������������

Figure 4:  Response Menu and short-cut options

You can now use any of the ten options 1-0 of the Response Menu to examine the

results. Try option 0 for a conventional plot of forward and reverse power gain

or attenuation (S21 and S12) against frequency, and option 9 for a plot of input

and output reflection coefficient (S11 and S22) against frequency.

The two display parameters are colour-coded in red and blue, but this filter is

a symmetrical network so the superimposed plots appear as a single purple line.

When you have finished, you can either press Esc to return to the Main Menu, or

press one of the short-cut options F or E to go directly to the other sub-menus.

(We suggest you continue reading at least as far as Section 10 before exploring

the Optimization sub-menu, which is described in Part B of this manual.)

To exit from SCALC, return to the Main Menu and press Q. In response to the

Confirm (y/N) message press Y to exit; any other key is interpreted as 'No'.

This ends our brief introductory tour. To make full use of SCALC, including the

powerful optimization feature, you first need to know more about SCALC networks

and .NET files.

4.    MAIN MENU FUNCTIONS

E  EDIT NETWORK

Enters the Network Editor (see Section 10).

O  OPTIMIZATION

See Part B, Sections 15-20.

F  FREQUENCY RANGE

Sets the frequency range over which a network is to be analysed

(see Section 12).

Q  QUIT TO DOS

Requires confirmation: pressing any key except Y returns you to the Main Menu.

A  ANALYSE NETWORK

See Sections 12 and 13. You must first have created or loaded a network, and

must also have set a frequency range using the F command.

I  INFORMATION ABOUT SCALC

Displays an information screen - either the Shareware message or a different

message for registered versions.

5.    SIMPLE PASSIVE NETWORKS

5.1   How SCALC works

SCALC's .NET files are used to describe the network that you wish to analyse or

optimize. SCALC reduces every network to a simple 'ladder' format. There is one

'series branch' running through from input to output (Figure 5) and optional

'parallel branches' between the series branch and ground. Each parallel branch

consists of a group of circuit elements, and the series branch is a chain of

similar elements. A parallel branch can join to the series branch at any

junction between elements, or at the input or output of the whole network.

<���������������  one series branch ������������>

�������Ŀ               �������Ŀ

�������������Ĵ       ���������������Ĵ       ��������

�       ���������       �       ���������

�����Ŀ                 �����Ŀ

�     � <- parallel ->  �     �

input     �     �    branches     �     �                output

�     �                 �     �

�������                 �������

�                       �

������������������������������������������������������

Figure 5:  Network composed of series branch and two parallel branches.

Note how the series branch continues after the second parallel

branch has been added.

When analysing or optimizing, SCALC works through the network from input to

output. It begins by calculating the S-parameters of the first two circuit

elements and combining them to give a new result, namely the S-parameters of

those two cascaded elements which are now considered as a single network. The

calculation then steps onwards, 'absorbing' circuit elements one by one and

calculating the S-parameters as it goes. When the program reaches the output and

stops, it has calculated the S-parameters of the whole network from input to

output - in other words SCALC has analysed the entire multi-element network.

You need to enter circuit elements into the .NET file in the correct logical

order to permit this stepwise analysis. You also need to tell SCALC how to

combine elements together, and where the calculation is to end.

Although SCALC works in terms of ladder networks, it can actually deal with

quite complex circuits (e.g. bridged-T networks which have two concurrent series

branches instead of just the one). The key to these operations is SCALC's

ability to calculate the S-parameters of a subcircuit and then treat the results

as a single new circuit element. If you present the data in the right order (and

we'll show you how), SCALC can simplify very complex circuits back down into

basic ladder networks. More details in Section 6.

5.2   Simple R/C/L elements

Let's introduce the simplest circuit elements first. These are individual R, C

or L components, as shown in Figure 6. Note that different codes are used for

the series and the parallel branches of the ladder network. Each element has a

single parameter value, in the normal microwave engineering units of ohms,

picofarads or nanohenries.

������������Ŀ                 �����������Ŀ

� ��������Ŀ �               �����������������

���ĴR,C or L�����               �    �� R,C�

� ���������� �                 �    � �or L�

�            �                 �    ���    �

������������������             �����������������

��������������                 �������������

RES  CPS  INS                  REP CPP INP

R C or L in series branch       R C or L in parallel branch

Figure 6:  Simple R, C and L elements

5.3   Cascading elements

The CAX command

The CAX (cascade inclusive) command tells SCALC how to analyse the network. CAX

has two parameters n1 and n2 corresponding to the first and last elements to be

cascaded in sequence. The range of elements from n1 to n2 inclusive must have

been defined ahead of the CAX command; SCALC will check that you have done this.

Referring back to Figure 3, our example file LPF.NET, the calculation begins at

element 1 and calculates the S-parameters of all following elements up to and

including element 5. Element 6, the CAX command, then contains the result of the

cascading operation and can be treated as a circuit element in its own right.

Since the next instruction is END (see Figure 3) that particular analysis is

complete because the S-parameters of the whole network have been calculated. In

a more complex circuit you can use CAX more than once to combine elements into

blocks - see Section 6.2 - and then use CAX one final time to calculate the

results for the entire network.

The CAS command

CAS n1 n2 is a special case of CAX which cascades just two elements, and the

results become the S-parameters of the element containing the CAS command

itself. Note that n1 and n2 do not need to be consecutive, but they must both

have been defined somewhere ahead of the CAS command; SCALC will check that you

have done this.

The END command

The command END n1 implies 'Display the results for element n1'. Note that END

does not have an element number of its own: n1 is the number of some previous

element - usually a CAX or CAS instruction.

END must always be the last instruction in an SCALC network file, though the

element number does not necessarily have to be that of the last element. This

useful feature allows you to alter the END instruction to look back at

intermediate stages of the analysis.

Sections 5.1, 5.2 and 5.3 have explained all the features of the LPF example

used in for the 'guided tour' in Section 3. Before trying to go any further,

please look again at the LPF.NET file described in Section 3 and make sure you

understand exactly why it was written that way.

5.4   Combined R/C/L elements

Section 5.2 described simple R, C or L elements. For convenience, SCALC also

allows you to describe small networks containing combinations of two R/C/L

components using a single element code. This is handy for including losses or

parasitic reactances in microwave components without taking up too many lines

in the .NET file. The relevant codes are shown in Figures 7 and 8.

�������������Ŀ                     �����������Ŀ

� ���Ŀ ���Ŀ �                   �����������������

���Ĵ 1 �Ĵ 2 �����                   �   ���Ŀ   �

� ����� ����� �                     �   � 1 �   �

�             �                     �   �����   �

�������������������                   �   ���Ŀ   �

���������������                     �   � 2 �   �

�   �����   �

�����������������

Dual series-connected                    �������������

elements in series branch               Dual series-connected

elements in parallel branch

1      2                                1     2

RLS     R      L                        LRS     L     R

RCS     R      C                        CRS     C     R

LCS     L      C                        CLS     C     L

Figure 7:  Codes for dual elements connected in series

�������������Ŀ                  �������������Ŀ

�   �����Ŀ   �                  �             �

� �Ĵ  1  �Ŀ �                �������������������

���Ĵ ������� �����                � ���Ŀ ���Ŀ �

� � �����Ŀ � �                  � � 1 � � 2 � �

� �Ĵ  2  ��� �                  � ����� ����� �

�   �������   �                  �   �     �   �

�������������������              �������������������

���������������                  ���������������

Dual parallel-connected           Dual parallel-connected

elements in series branch         elements in parallel branch

1      2                                1     2

RLP      R      L                        LRP     L     R

RCP      R      C                        CRP     C     R

CLS      C      L                        CLP     C     L

Figure 8:  Codes for dual elements connected in parallel

6.    ADVANCED PASSIVE NETWORKS

This section explains the additional commands and file structures that can be

used to model quite complex passive networks which are initially far removed

from the basic topology of a ladder network.

You've already seen how to combine elements using CAX and CAS to create a simple

network. The next step is to use CAX and CAS to combine elements into

building-blocks, which in turn are combined using further CAX/CAS instructions

to build more complex networks.

To increase the potential scope of SCALC even further, you can also save

intermediate results as files of frequency-dependent S-parameters, which you can

then use in analyses of more complex networks still.

This section introduces additional network-building instructions, and completes

SCALC's library of basic circuit elements.

6.1   Repeating networks using EQU and IMA

Many networks have features which either repeat consecutively or have

mirror-image symmetry. SCALC makes these very easy to analyse.

The EQU command

EQU is one of the simplest commands. It has one numerical parameter (n1), and

simply says 'Make this circuit element the same as element n1 was'. For example,

the command 9 EQU 7  makes element 9 the same as element 7.

For example, if element 7 was a CAX 1 6 command, then 9 EQU 7 has duplicated

elements 1-6 inclusive using just one command.

The IMA command

IMA is similar to EQU except that instead of a duplicate it produces a

mirror-image of an element, interchanging the input and output ports. IMA can be

extremely useful in filter networks, which often tend to be symmetrical about

their half-way point.

6.2   Transmission lines

The basic transmission line element in the series arm is TRL (Figure 9).

Parameters are: characteristic impedance (ohms); electrical length (degrees,

where 360 deg = 1 wavelength); and the frequency (GHz) at which the electrical

length applies.

Transmission lines in the parallel arm are called 'stubs', OST for an open-circuit stub

and SST for a shorted stub (Figure 10). Parameters are the same as for TRL.

����������Ŀ

Transmission line        ����������������

TRL                   �          �

����������������

Impedance Z�, �� long at F GHz    ������������

Figure 9:  The series transmission line element

����������Ŀ                 ����������Ŀ

����������������             ����������������

�    ��    �                 �    ��    �

�    ��    �                 �    ��    �

����������������             ����������������

������������                 ������������

OST                           SST

Open circuit stub            Short circuit stub

Both have impedance Z�, �� long at F GHz

Figure 10:  Open and short-circuit stubs

The CPL element comprises coupled parallel transmission lines in the series

branch (Figure 11). These lines are characterized by their even-order and odd-

order impedances Zoe and Zoo, and the frequency at which the lines are 90� (a

quarter-wavelength) long. For further details of the use of Zoe and Zoo in the

design of couplers and filters, see texts on filter synthesis.

����������Ŀ

����������� ����

�         ��

�  �������ٳ

�          �

����������������

������������

CPL

Zoe, Zoo, F GHz at which lines are 90� long

Figure 11:  Coupled lines

Finally in this 'transmission lines' section we have short and open circuits, SHT and

OPN, and an idealized transformer TFR (Figure 12).

����������Ŀ                      ����������Ŀ

���Ŀ      �����                  ����        ����

� �      � �                      �          �

� �      � �                      �          �

� �      � �                      �          �

����������������                  ����������������

������������                      ������������

SHT                                OPN

Short circuit                       Open circuit

����������Ŀ

�����Ŀ  �������

�   )  (   �

�n1 )  ( n2�

�   )  (   �

����������������

������������                                      TFR

Perfect transformer, ratio n1:n2

Figure 12:  SHT, OST and TFR

6.3   Non-ladder networks using SER and PAR

What if your network doesn't look anything like a ladder? SCALC contains two

instructions, SER and PAR, which allow you to convert such networks into the

required ladder configuration.

Here's a simple example. This lossy short-circuited stub (Figure 13) has just

one parallel branch, but does not conform to the requirements for a ladder

network because that branch contains two components in series. The conversion is

very simple, as shown below in the listing of LOSSTUB.NET.

Network representation          Circuit

������������������������       ����������

�                       �

�������Ŀ                  ��

�  REP  �                  � �

�       �                  � �  10R

�  10R  �                  ���

���������                   �

�                       �

�������Ŀ                  |�|

�  SST  �                  |�|

�  90�  �                  |�|

�       �                  |�|  quarter

�       �                  |�|    wave

���������                  |�|    line

�                      |�|

�                  ����|�|

������������������������      ������������

Element No.   #1       #2       #3

SST   1    50�      90�lng   1GHz

REP   2    10�

SER   3    1el#     2el#

END   3

Figure 13:  Lossy short-circuit stub, with circuit diagram and

listing of LOSSTUB.NET

The SER instruction combines the two elements SST and REP. Element 3 containing

the SER instruction then represents the complete lossy stub.

Figure 14 shows a different situation, where the series branch is doubled. The

listing is from file DIPEX0.NET. To reduce this bridged-T diplexer to a single

series branch requires several steps, as shown in the network breakdown on the

next page.

��������Ĵ��������������������������Ŀ

�      6-CPS             7-TRL       �

�                                    �

�    ______               ______     �

��������Ĵ  50R �������������Ĵ  50R ������������

------       �       ------

1-RES        �       4- RES

���������Ŀ

�         �

�         �

�         �

���        �

3-CPP   ���        �  2-SST

�         �

�         �

�����������

�

�������������������������������������������������

Element No.   #1       #2       #3

RES   1    50�

CPP   2    3.18pF

SST   3    50�      45�lng   1GHz

RES   4    50�

CAX   5    1el#     4el#

CPS   6    3.18pF

TRL   7    100�     45�lng   1GHz

CAS   8    6el#     7el#

PAR   9    5el#     8el#

END   9

5  CAX                8 CAS               9 PAR

����������Ŀ          ���������Ŀ         ���������Ŀ

Ĵ          ��        Ĵ         ��      �Ĵ    8    �Ŀ

�  1 to 4  �          �  6 + 7  �       � ����������� �

�          �          �         �       � ���������Ŀ �

������������          �����������     ���Ĵ    5    �����

�                     �              �����������

///                   ///                  �

///

Figure 14:  Circuit diagram, SCALC listing and network breakdown of

DIPEX0.NET, a bridged-T diplexer containing two concurrent

series branches.

This quite complex circuit is dealt with as shown in the network breakdown

above. First, the bottom part of the T-network (elements 1 to 4) is combined

into a single element 5 using the CAX instruction. Likewise the 'overbridge'

(elements 6 and 7) is combined using CAS to form element 8. This leaves us with

the two elements 5 and 8 which are paralleled along the series branch. The new

instruction PAR combines these into the single element 9, which in this case

comprises the complete network.

Almost any two-port network you're likely to find in microwave circuitry can be

reduced to a simple ladder network by means of SER and PAR.

7.    S-PARAMETER (2-PORT) FILES

SCALC can both read and write files of two-port S-parameters. Thus any circuit

element can be treated as a 'black box' characterized by its S-parameters ad a

function of frequency (Figure 15). S-parameter files all have the .DAT filetype

extension, and are called into network listing using the TWO command. Instead of

any component parameters, TWO simply requires a filename. (As usual in SCALC,

you must NOT enter the .DAT extension.)

����������Ŀ

�Ĵ   >S21>  ���

port 1  �S11    S22�  port 2

�Ĵ   <S12<  ���

������������

TWO

Figure 15:  The TWO network element - a file of frequency-dependent

S-parameter data

Figure 16 shows the listing of MSA0104.DAT (obtained using the Datafile Editor -

see Section 11 later). All SCALC two-port datafiles have the same format; each

line contains:

Frequency (GHz), S11 (magnitude -not dB- and angle), S21, S12 and finally S12.

FREQ    S11     S21     S12     S22  � FREQ    S11     S21     S12     S22

GHz   mag �   mag �   mag �   mag � �  GHz   mag �   mag �   mag �   mag �

0.1   .06 141 8.3 170 .08 5   .07-9    2.5   .08-20  2.7 46  .21 25  .05-112

0.2   .08 112 8.0 160 .08 9   .07-15   3.0   .02-37  2.4 31  .23 18  .05-134

0.3   .1  94  7.7 151 .08 15  .07-22   3.5   .05 128 2.1 15  .25 10  .06-160

0.4   .12 77  7.3 142 .08 16  .07-32   4.0   .12 113 1.9-1   .27 2   .06-175

0.5   .13 70  7.0 134 .09 19  .07-37   4.5   .19 97  1.7-15  .29-7   .07 173

0.6   .14 56  6.6 127 .09 21  .08-44   5.0   .27 80  1.5-30  .29-15  .07 150

0.8   .16 41  5.8 114 .11 27  .08-53

1.0   .17 28  5.2 102 .11 29  .08-61

1.5   .17 5   4.0 78  .15 30  .08-73

2.0   .13-12  3.2 59  .18 25  .07-90

Figure 16:  Listing of MSA0104.DAT

If you want to save the S-parameters of a network you have just analysed, this

can be done using option 5 of the Response Menu (Figure 4, and Section 13.5).

You can then read that file back into another network listing, so that larger

and more complex networks can be analysed.

When exiting the Network Editor to begin an analysis, SCALC will interpolate the

file data to cover the frequency range you require.

CAUTION Do not try to use any file data outside their valid frequency range.

Also, two-port data of a network containing resonant circuit elements may have

S-parameters which change extremely rapidly with frequency, and may defeat

SCALC's interpolation routines. Always keep resonant elements within the main

analysis.

8.    OTHER TYPES OF DATAFILES

As well as reading and writing S-parameter files, SCALC can read files of

frequency-dependent data in so-called 'one-port' format. Actually there are

three different types of files, all sharing a similar format.

Figure 17 illustrates how IMS and IMP files are used in networks.

����������Ŀ              �����������Ŀ

���������Ŀ�            �����������������

��Ŵ  R+jX  ����            ����������Ŀ�

����������ٳ              ��  R+jX   ��

�          �              �����������ٳ

����������������          �����������������

������������              �������������

IMS                        IMP

Impedance in series branch    Impedance in parallel branch

Figure 17:  IMS and IMP network elements - files of frequency-dependent

resistances and reactances

Both types of files are in the same line format: frequency, real part of

impedance, imaginary part. This format is repeated for all the frequencies

concerned. The real and imaginary parts are assumed to be connected in series.

Although IMS and IMP are designated by SCALC's Network Editor (Section 10) as

'one-port' formats to distinguish them from the S-parameter format used by TWO,

the data supplied in IMS and IMP files are used internally to calculate

S-parameters for genuine two-port series and shunt network elements as indicated

in Figure 17.

Figure 18 shows the ONE network, a true one-port which is characterized only by

its complex reflection coefficient � (gamma). Each line in a ONE file consists

of frequency, the magnitude �� of the reflection coefficient, and the phase

angle ��. This line format is repeated for all the frequencies concerned.

�����������Ŀ

�������Ŀ     �

����������Ŀ�

�� �� + ��

�����������ٳ

���������     �

�������������

ONE

One-port network

Figure 18:  The ONE network element - a file of frequency-dependent

reflection coefficient data

To repeat the warning from Section 7: When exiting the Network Editor to begin

an analysis, SCALC will interpolate the file data to cover the frequency range

you require. Obviously you should not try to use any file data outside their

valid frequency range. Avoid the use of IMS, IMP and ONE data files which change

extremely rapidly with frequency, for this may defeat SCALC's interpolation

routines.

9.    CODEWORDS CLASSIFIED

9.1   Introduction

You have now been introduced to almost all of the SCALC network codewords. This

section contains a reference list which classifies the codewords in a different

way, as Components, Transformations or Interconnections. The same classification

is used in the Help window of the Network Editor (press F1).

'Components' are self-evident: they are circuit elements like real-life

components, and are all characterized by at least one parameter value in ohms,

picofarads, nanohenries etc. (except of course for the short- and open-circuit).

See Section 9.2 and Figures 19-30.

'Interconnections' are the ways of grouping Components together to make a

network or a sub-network. See Section 9.3.

'Transformations' operate on Components or sub-networks and are ways of changing

them into another type. See Section 9.4.

The remaining SCALC codewords are listed in Section 9.5.

NOTE - All the necessary information is provided by the Help function of the

Network Editor. Sections 9.2 to 9.5 (including Figures 19-40) are available

only in the fully illustrated printed manual, which you will receive on

registration of SCALC.

10.   NETWORK EDITOR

10.1  General description

The SCALC Network Editor is used to create networks for subsequent analysis.

The screen is divided into three parts (Figure 2). The whole network (if any) is

listed in the middle of the screen, with the current line expanded for editing

at the upper left. The menu of Network Editor commands is shown in the lower

part of the screen.

Information about each network element is shown in the following way:

ELEMENT TYPE      The three-letter code that describes the type of element or

command. For help on these codes, press the F1 key.

ELEMENT NUMBER    The reference number of the element. In most cases, this

must be a unique number which cannot be re-used.

END is a special case where the element number 'belongs' to

a previously-defined element (usually a CAS or CAX

instruction).

PARAMETER #1      Information concerning the element. SCALC will ask for as

(or a FILE NAME)  many parameters as the particular type of element requires.

PARAMETER #2,     Units are shown and prompted for on entry.

PARAMETER #3

10.2  Network Editor commands

All of the commands available are shown in the menu displayed at the bottom of

the screen (Figure 2).

All commands relate to the 'current line' which is generally shown highlighted.

The top left-hand corner of the screen shows the current line number, with

details of the element concerned. If no line is highlighted, the current line

will be added at the end of the network.

Press the up-arrow or down-arrow key to change the current line, and the

highlight moves accordingly. If no line is highlighted, you must be at the end

of the network. If you go beyond the end of the network, the highlight 'scrolls'

around to the beginning; conversely if you go back before the beginning, the

highlight disappears because you have in fact moved around to the end.

Whenever you are not involved in executing a Network Editor command, you can

also press the F1 key to see Help information about the available network codes.

The codes appear with brief descriptions in a window at the bottom of the

screen. Press the PageUp or PageDown keys to scroll through the information, or

Esc to return from the Help function to the Network Editor.

The Network Editor commands are:

I  INSERT LINE

Inserts a line ABOVE the current line position (or at the end of the network).

You will then be prompted for the element code, element number and any necessary

parameters. New data appear in blue as you type, and change to white after you

press the Enter key. Line numbers (as displayed at the top of the screen) will

update automatically after each insertion or addition.

M  MODIFY LINE

M allows the parameters of the element on the current line to be modified. The

element type cannot be modified, and nor can its element number - to change to a

different type of element or renumber an element you must delete it (see below)

and insert a new one. New parameter values overtype the old data in blue, and

change to white on pressing Enter.

D  DELETE LINE

Deletes the highlighted line. Lines below move up and are renumbered, but

element numbers and cross-references stay the same.

P  PRINT NETWORK

Lists the network to your printer, if the printer is on-line.

S  SAVE NETWORK

Saves the network to a file. A filename prompt appears at the top right of the

screen, with the last-used network filename as a default. Press Enter to accept

this filename, or enter a valid DOS filename of up to eight characters. The

suffix .NET is added automatically by SCALC: do NOT enter it yourself.

If a file of the same name already exists (as it will if you accept the default

filename) you will be prompted with a beep and a message at the top right of the

screen before SCALC overwrites the existing file. You must press Y to overwrite

the existing file; press any other key to escape from this function without

making any changes.

L  LOAD NETWORK

Loads a new network from a file. If there is a network already in memory, you

will be prompted with a beep and a message at the top right of the screen before

SCALC overwrites it. You must press Y to erase the network and proceed; press

any other key to escape from this function without making any changes.

If you press Y, a filename prompt appears at the top right of the screen, with

the last-used network filename as a default. Press Enter to reload the last-used

file, effectively canceling any changes you might have made since the previous

time you loaded it. If instead you start typing a filename, the default filename

is cleared and you can enter a valid DOS filename of up to eight characters. The

suffix .NET is assumed automatically by SCALC: do NOT enter it yourself.

C  CLEAR NETWORK

Erases the network from memory without saving anything. You will be prompted

with a beep and a message at the top right of the screen, asking for

confirmation. Press Y to go ahead, or any other key to escape without erasing

the network.

E  DATAFILE EDITOR

See Section 11.

F  NETWORK FILE CATALOG

Displays a list of the .NET files in the same directory as SCALC.EXE. Press

Enter to return to the Network File Editor.

Esc  EXIT NETWORK EDITOR

See Section 10.3 below.

10.3  Network checking

Press Esc when you have finished entering or loading your network. SCALC will

check that the network for validity, and will only return you to the Main Menu

if the network is OK.

The following checks are made:

1. All elements are checked for duplication, to ensure that each number is

unique.

2. All interconnections and transformations are verified. The elements which

are to be operated on are checked to confirm that they do in fact exist in

the network.

3. Any elements specified via filenames (see Sections 7 and 8) are checked to

confirm that the relevant files are present in the same subdirectory as

SCALC itself.

4. Any external data files are retrieved and a data fit is carried out

(see Sections 7 and 8).

If the network is not valid, SCALC will sound a warning beep and highlight the

first incorrect line in red. You cannot exit from the Network Editor until SCALC

has confirmed that the network is valid (or until you have erased the network

using the C command).

TIP -  If you cannot see why the network is not valid, and don't have time to

solve the problem right now, you can save the faulty network in a

temporary .NET file using the S command; then press C to clear the

network, which allows you to exit the Network Editor using Esc.

11.   DATAFILE EDITOR

The Datafile Editor is used for writing and editing files used for TWO, ONE,

IMS, and IMP commands (see Sections 7 and 8).

11.1  One-port or two-port data

When first entering the Datafile Editor from the Network Editor, and when

starting a new file from  within the editor, the program will ask whether you

are going to supply data in '2-port' form. Only press Y if you are going to

enter true two-port data for use with the TWO command. Pressing any other key

will switch the Datafile Editor into its one-port/impedance mode for use with

IMS, IMP and ONE commands (see below and Sections 7 and 8).

Figure 41 shows the input screen for two-port data. To enter or modify data,

follow the on-screen prompts. Wherever possible the Datafile Editor follows the

same conventions as the Network Editor already described in Section 10.

�����������������������������������������������������������������������������Ŀ

Line number 1     Frequency:                                  Datafile Editor

|S11|              /S11

|S21|              /S21

|S12|              /S12

|S22|              /S22

FREQ    S11     S21     S12     S22  | FREQ    S11     S21     S12     S22

GHz   mag �   mag �   mag �   mag � |  GHz   mag �   mag �   mag �   mag �

I - Insert line   P - Print data   C - Clear data         _ - Line up

M - Modify line   S - Save file    G - Graph data         _ - Line down

D - Delete line   L - Load file    F - Datafile catalog  Esc - Done

�������������������������������������������������������������������������������

Figure 41:  The Datafile Editor in two-port mode

The frequency is always entered in GHz, and is the first item in every data

entry. Data may be entered for up to 20 frequencies, and must be arranged in

ascending order of frequency.

11.2  Two-port data format

Two-port data - namely the S-parameters S11, S22, S21 and S22 - can only be used

in conjunction with the TWO command. Data are always entered as magnitudes (not

dB), normalized to 50� and in polar form. Follow the on-screen prompts.

If you read in two-port data using the L command, the display will look

something like Figure 16, which is the datafile for the MSA01014 MMIC. Note that

the editing highlight will be on the first line.

11.3  One-port data formats

One-port data can be used with the ONE, IMS and IMP commands, but each command

requires data in its own correct format.

The ONE command requires frequency-dependent reflection coefficients in the same

convention as S11 above. The resulting impedances are assumed to be inserted in

the parallel branch of the network - in other words, connected from the network

element's single port to ground.

The IMS and IMP commands both require frequency-dependent data in the form of

series-connected resistances and reactances in ohms. IMS inserts these

impedances in the series branch, and IMP inserts them in the parallel branch to

ground.

11.4  Graphical review

One option in the Datafile Editor which is not present in the Network Editor is

G - Graph Data. For example, if you load the datafile MSA0104(.DAT) as shown in

Figure 16 and then press G, SCALC generates a sequence of two colour graphics

displays (Figures 32 and 33).

Figure 42 is the Smith chart display of S11 (blue) and S22 (red), for the

frequency range in question. Notice the white highlight at the low-frequency end

of the S11 trace. You can step this highlight along using the up-and down-arrow

cursor keys, and the white display at the bottom of the screen will change to

show frequency, magnitude and phase angle. When you have finished inspecting the

S11 data, press Enter and the movable highlight is transferred to S22.

[Sorry - this figure can only be displayed in the printed manual]

Figure 42:  Smith chart display of S11 and S22 for MSA0104

Press Enter again and the display changes to a rectangular frequency plot of S21

(blue) and S12 (red) as shown in Figure 43 Both S-parameters are plotted in dB,

without phase angles.

[Sorry - this figure can only be displayed in the printed manual]

Figure 43:  Rectangular display of S21 and S12 for MSA0104

Finally, press Enter a third time to return to the Datafile Editor.

11.5  Saving datafiles to disk

Data files are intended to be saved on disk using the S command before exiting

the Datafile Editor, as the data temporarily held in memory will be lost.

Therefore SCALC will sound a warning beep and ask for confirmation before you

can exit the Datafile Editor.

12.   ANALYSIS - SETTING FREQUENCY RANGE

To determine the frequency response of a network, you obviously have to specify

a frequency range. Up to 20 frequencies can be specified, and these will be the

data points for both graphical and tabular output.

There are two ways to enter the frequency range - an automatic 20-point sweep or

manual entry of up to 20 frequencies. In most cases automatic sweep is

preferable, though manual sweep is better if you want output at some specific

'odd' frequencies.

To enter a frequency range, press F from the Main Menu (Figure 1), or from the

Response Menu if you have already completed an analysis and wish to change the

frequencies. In both cases the display at the top right of the screen changes

to Frequency Range, and a Start Frequency (GHz) :  prompt appears.

Enter a start frequency. A frequency of zero is not accepted because it will

produce computational errors or other non-meaningful results. When your first

frequency is accepted, it is transferred to the Previous Frequency line and a

new prompt appears (Figure 44).

Previous frequency  1.000  GHz

Frequency #  2  (GHz) :

Figure 44:  Prompt for entry of second frequency

If you enter a frequency which is less than or equal to your previous entry,

SCALC will switch into automatic-sweep mode. Using your last entry as a

frequency interval, SCALC will automatically complete the set of 20 frequencies

and display the 20th value as the Stop Frequency. The following table shows a

few examples.

-----------INPUTS------------       ---------RESULTING SWEEP---------

Freq #1    Freq #2    Freq #3       Start freq   Interval   Stop freq

1.0        1.0         n/a            1.0          1.0         20.0

3.0        0.1         n/a            3.0          0.1          4.9

0.1        0.3         0.2            0.1          0.2          3.9

For manual entry you can enter up to 20 frequencies in increasing order, at any

frequency interval. Continue until you have entered as many frequencies as

desired, up to a maximum of 20. You can terminate frequency entry at any time

by pressing only Enter in response to the next input prompt.

If at any time you manually enter a frequency which is less than or equal to

your previous entry, SCALC switches into automatic sweep mode to complete the

set of 20 frequencies. (This is probably not what you wanted!)

After a few moments the display of frequencies clears and you are returned to

the Main Menu.

Note: If you have entered some optimization objectives (see part B of this

manual) or loaded them from a file, you will already have loaded some

frequencies into memory which will then become the default frequencies for

analysis.

CAUTION

If you then alter the frequency range for analysis, you will need to

re-load your objectives before you can do any further optimization.

13.   RESPONSE MENU

From the Main Menu, press A to begin the analysis. The message at the top right

of the screen changes to Analysis in progress. If the network is complicated

and you don't have a math coprocessor or 486DX, SCALC may offer you a few

moments to contemplate investing in a faster number-cruncher...

Eventually the message changes to Analysis Complete and SCALC presents the

Response Menu (Figure 4). You have ten output options, selected by pressing 1,

2... or 0, and described in sections 13.1 to 13.10 respectively.

13.1  Table of reflection S-parameters

A table of S11 and S22 (magnitude and angle) at the frequencies selected for

analysis. Press Enter to return to the Response Menu, or P to print the table

(if your printer is on-line).

13.2  Table of transmission S-parameters

A table of S21 and S12 (magnitude and angle) at the frequencies selected for

analysis. Press Enter to return to the Response Menu, or P to print the table

(if your printer is on-line).

13.3  Table of input/output impedances

A table of input and output impedances in (R+jX) form at the frequencies

selected for analysis. Press Enter to return to the Response Menu, or P to

print the table (if your printer is on-line).

13.4  Table of S-parameters in dB

A table of all four S-parameters (S11, S21, S12 and S22) in decibels but

without phase angles, at the frequencies selected for analysis. Press Enter to

return to the Response Menu, or P to print the table (if your printer is

on-line).

13.5  Save S-parameters to data file

This option will save the S-parameter results for the whole network at the

analysis frequencies specified, into an SCALC two-port .DAT file. The file has

the same format as any other SCALC two-port datafile. It can be read back into

the Datafile Editor (Section 11, Figure 41) and allows the network you have

just analysed to be used as part of a larger network still.

The default filename is the same as that of the network you have just analysed.

Press Enter to accept this default, or overtype it with a new filename. Do NOT

enter the suffix .DAT - SCALC does this for you.

13.6  Table of input stability circles

This table is only meaningful for networks containing an active device. See

Section 20 for further explanation of the format. Press Enter to return to the

Response Menu, or P to print the table (if your printer is on-line).

13.7  Table of output stability circles

This table is only meaningful for networks containing an active device. See

Section 20 for further explanation of the format. Press Enter to return to the

Response Menu, or P to print the table (if your printer is on-line).

13.8  Smith chart of S12 and S22

This option produces a Smith chart display of S11 (blue) and S22 (red), for the

frequency range in question. The appearance and functions are similar to those

of the Graph data option of the Datafile Editor (Figure 42) but note that

symmetrical passive networks will show only a single purple line because S11

and S22 are coincident.

Notice the white highlight at the low-frequency end of the S11 trace. You can

step this highlight along using the up-and down-arrow cursor keys, and the

white display at the bottom of the screen will change to show frequency,

magnitude and phase angle. When you have finished inspecting the S11 data,

press Enter and the movable highlight is transferred to S22.

13.9  Graph of reflection S-parameters

This option produces a rectangular frequency plot of S11 (blue) and S22 (red),

similar to Figure 43. Both S-parameters are plotted in dB, without phase

angles. Symmetrical passive networks will show only a single purple line

because S11 and S22 are coincident. Press Enter to leave this menu function.

13.10  Graphs of transmission S-parameters (S21 and S12)

This option produces a rectangular frequency plot of S21 (blue) and S12 (red),

similar to Figure 43. Both S-parameters are plotted in dB, without phase

angles. Symmetrical passive networks will show only a single purple line

because S21 and S12 are coincident. Press Enter to leave this menu function.

13.11  Main Menu shortcuts

When you leave the Response Menu, you do not always need to pass through the

Main Menu to reach the next function you need. As shown in Figure 4, you can

take the following single-key shortcuts:

F  - set a new frequency range

E  - go directly to the Network Editor

O  - go directly to the Optimizer

- or press Esc to return to the Main Menu.

End of Part A

##################################################################################################################

#